Plan View

186 m

0 m

360 m

FBP X

FBP Y

FBP Z

FCS X

FCS Y

FCS Z

Fire line

Road

360 m

160 m

- Experimental results

|  | FCS Z | FCS Y | FCS X |
| --- | --- | --- | --- |
| Firebrand density (pcs/m2) | 536 | 463 | 335 |
| Time span (s) | 394 | 513 | 407 |
| Total firebrand flux (pcs/m2.s) | 1.361 | 0.902 | 0.824 |

Let’s take FCS Z firebrand collection plot:

- 536 firebrands landed on 1m2 in 394 seconds.

- Simulation results

|  | FCS Z |
| --- | --- |
| Firebrand density (pcs/m2) | 243 |
| Time span (s) | 175 |
| Total firebrand flux (pcs/m2.s) | 1.388 |

Estimated number of firebrands in 394 seconds =243175 ×394

= 547

Fire intensity and input number of firebrands

Fireline length			=130 m

Fire line depth			= 2 m

Fireline intensity 		=18.46 MW/m

Firebrands input volume 	= 2m x 130 m x 9 m = 2340 m3

Input number of firebrands	= 8160 pcs/s

Input number vs intensity	= (8160 pcs/s)/[(18.46 MW/m)] ≈ 442 pcs/MJ.m